| # INDUSTRIAL LIFTING (UK) LTD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit 24 Parkhall Business Village Telephone: 07710 992098 Parkhall Road Development Member Telephone: 01782 595945 Stoke on Trent Fax: 01782 595994 ST3 5XA Email: accounts@ilgukltd.com |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | ## Company Registration No: 4142350 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Southern Gate Lift 1 Chichester Sussex PO19 8SG |  |  |  |  |  |  |  |  |  | Orona Ltd XGB62426LR 2017 CA 000279 O/No 009669 T. McGowan |  |  |  |  |  |  |  |  |  |  |  |
| TEST CERTIFICATE NUMBER: 22620/5218/A |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Plant No. | Description |  |  |  |  |  |  |  |  |  | Qty |  | Safe Working Load |  | Proof Load |  |  | Date of test/ examination |  | Date of next thorough examination |  |
| 5218-1 | Lift shaft overhead runway beam |  |  |  |  |  |  |  |  |  | 1 |  | 2000kg |  | 2500kg |  |  | 5.2.18 |  | 5.2.19 |  |
|  | This is inserted into the wall at each end |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Overall length 1700mm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ### Section Size 152mm x 152mm Universal Column |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ### Safe working load deflection nil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ### This has one lifting eye fixed to the underside of the beam with a SWL 2000kg ### Serial number LP02 ### This has two harness points fixed to the underside of the beam with a SWL 300kg ### Serial numbers HP01 and HP02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tick box For applicable L.O.L.E.R. Requirement |  |  |  √ First known thorough examination |  |  Regulation 9.(3)A(i) 6 months |  |  Regulation 9.(3)A(ii) 12 months |  |  |  |  |  Regulation 9.(3)A(iii) examination |  |  |  |  Regulation 9.(3)A(iv) after repairs |  |  |  √ Equipment safe to Operate |  |  |
| Date: 5.2.18 |  |  |  | Signature: |  |  |  |  | Print |  |  |  |  | P ELLIS |  |  | Approved Quality Control |  |  |  | √ |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Approved Test/Examiner |  |  |  | √ |
| I certify that the lifting equipment described in this certificate has been tested and/or examined, and that the above particulars are correct. The above equipment has been tested and/or examined for normal use by a competent person in accordance with L.O.L.E.R. regulations 1998. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |