| Material: Swarfega Jizer Supplier: DEB Ltd, Denby Hall Way, Denby, DE5 8JZ |  |  |  | Date of MSDS: 09/05/15 Contents: Distillate Petroleum Hydrocarbon Light |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Health: Exposure Limit: Not determined YEL / WEL: |  |  |  | Method of Use: Hand applied using rag. Area: Well ventilated area adjacent to shaft. Exposure time: < 1 hour per day |  |  |  |
| Hazards: |  |  |  | Personal Protective Equipment: |  |  |  |
| Harmful |  |  |  | Gloves | Safety Glasses |  |  |
| Hazard Statements: R41: Risk of serious damage to eyes R65: Harmful: may cause lung damage if swallowed R66: Repeated exposure may cause skin dryness or cracking |  |  |  | Spillage: Do not discharge into drains, watercourses or on to the ground. Absorb with dry sand or earth – place in containers. |  |  |  |
| First Aid: Inhalation – Move affected person to fresh air at once – seek medical attention. Skin contact – Remove the contaminated clothing – wash skin with soap & water – seek medical attention if symptoms occur after washing. Eye contact – Irrigate eyes with plenty of water for at least 15 minutes, lifting eyelids – seek medical attention. Ingestion – If swallowed do not induce vomiting – seek medical advice immediately. |  |  |  | Handling: Avoid spillage, skin and eye contact. Do not smoke when handling. Personal hygiene to be maintained – wash hands before eating, smoking and using the toilet. |  |  |  |
| Fire: Extinguish with Foam, CO2, Dry Powder or Water Fog. During combustion toxic gases are produced (CO, CO2) |  |  |  | Storage: Store in tightly closed original container in a cool, well ventilated area. Avoid storing near strong oxidisers. |  |  |  |
|  |  |  |  | Disposal: Waste to be treated as controlled waste. Dispose to licenced waste disposal site in accordance with local authority. |  |  |  |